COMPUTER NETWORKS

- The number of networks possible in Class-A are 2n-2 and in Class B, C are 2n (n is number of bits for network). In class A, 2 is subtracted because of loop back address (127.x.y.x) and DHCP Client (0.0.0.0).

- A NAT router converts private IP to public IP address, but a private address is assigned using the DHCP protocol to each device on your network. The devices on your network will communicate with each other using their private IP address, and the router will use its public IP address to facilitate communication over the internet. When connecting to the internet, your private IP addresses will remain hidden, and only the routers public IP address is visible.

- The IP address is the address of the connection, not the host or router, because if the device is moved to another network, the IP address may be changed.

- The data link control sublayer deals with all issues common to both point to point and broadcast links and the medium access control sublayer deals only with issues specific to broadcast links.

- Go-back-N ARQ supports both individual as well as cumulative acknowledgements but accepts only inorder frames.

- Stop N wait ARQ supports out of order frames and only individual acknowledgements.

- Outstanding frames: The frames that are transmitted on the channel and waiting for the ack.

- In GBN an ackNo is cumulative; it defines the sequence numbers of the next packet expected, confirming that all previous packets have been received safe and sound. Th semantics of acknowledgement is different in SR. In SR, an ackNo defines the sequence number of a single packet that is received safe and sound.Assume a sender sends 6 packets 0, 1, 2, 3, 4, 5 and the sender receives an ACK =3. In GBN this means that packets 0,1,2 have been received whereas in SR, the ack=3 indicates that packet 3 has been received.

- Note that the IP addresses and the MAC addresses are not in the same order. For IP addresses, the source address comes before the destination address and for MAC adress, the destination address comes before the source.

- The right-most bit in the first octet (the least-significant bit in the most-significant octet) of a multicast MAC address has been reserved as binary "1".

- HDLC implements the Stop and wait protocol.

- The standard ethernet (10Mbps) chooses CSMA/CD with 1-persistent method.

- In CSMA/CA (wireless) we have two models, peer to peer model (or Ad Hoc model) and Infrastructure model.

In a peer -to-peer model or ad hoc mode of transmission, every node is going to use its own resources to transmit the data. Whereas in infrastructure mode of transmission, every node is going to use the resources for the server. Infrastructure mode is more practical.

- Contention Protocols resolve a collision after it occurs.These Protocols execute a collision resolution protocol after each collision eg. Pure/Slotted Aloha, CSMA/CA(wireless), CSMA/CD(ethernet).

- A contention period or contention slot is the minimum time a host must transmit such that it can be sure that no other host packet has been transmitting. For LAN using  CSMA/CD ,Contention period  must be twice the propagation delay (2*Tp).

- Collision can be detected in a wired medium because when a collision occurs in a wired medium, the energy almost doubles.

- Collisions can not be detected in a wireless medium because a collision only adds about 5 to 10 percent additional energy(which may not be a decisive factor for a collision).

- In Bus topology we use Manchester encoding and in Ring topology we use differential Manchester encoding.

- In case of fragmentation, the host or router that fragments a datagram must change the values of these fields : MF, DF, Fragment offset and total length, Checksum and TTL.

- The checksum must be recomputed at each router as the TTL field and possibly the option field as well, may change.

- ICMP error messages are transmitted not only by the mediating routers but also by the destination host (in case the destination does not get all the fragments, some fragments missing).

- ICMP is neither error detecting nor error correcting protocol. It is an error reporting protocol.

- No ICMP error message will be generated in response to a datagram carrying an ICMP error message.

No ICMP error message will be generated for a fragmented datagram that is not the first fragment .

No ICMP error message will be generated for a datagram having a multicast address.

No ICMP error message will be generated for a datagram having a special address such as 127.0.0.0 or 0.0.0.0.

- In case of Redirection message by ICMP, the packet is not dropped (bas icmp k through vo host ko bata deta hai ki agle baar dusre route se bhejna yaha se mat bhejna. Abhi jo bheja hai usko ye wala router bhej dega aur ICMP bhej dega host ko taki agli baar dusre route se bheje).

- The program ping is one program that uses ICMP to determine whether a system is connected to the Internet.

### ICMP packet does not have source and destination port numbers ICMP relates to the Internet Layer, whereas port numbers are only found in the Transport Layer, which is the layer above.

- The IP addresses are designed to work with stationary hosts because part of the address defines the network to which the host is attached.

- Scaling factor for the header length field is 4. If the HLEN is given as 10 then the actual header length will be : 4x10= 40 Bytes.

Length of data = total length - (HLEN) x 4.

- In the distance vector routing, the nodes only share the distance vectors(array of distance values) with their neighbours, not the whole routing table.

- In the Data link layer, we attach sequence number for every frame whereas in case of the TCP in transport layer, we attach the sequence number for every byte in the segment.

- The value in the sequence number field of a segment defines the number assigned to the first byte contained in that segment.

- The urgent pointer in TCP defines a value that must be added to the sequence number to obtain the number of the last urgent byte in the data section of the segment.

- The initial sequence number in case of TCP will always be a random number within the range of 0 to 232-1 to provide protection to the data.

- The SYN segment does not carry any data but it consumes one sequence number.

- TCP can accept out of order segments but always send in order acknowledgements. TCP can send back to back segments without waiting for the ack. If Ack are lost, TCP will not bother about the ack because the next upcoming ack will nullify the previously lost ack.

- Data may arrive out of order and be temporarily stored by the receiving TCP, but TCP guarantees that no out of order data are delivered to the process i.e. application layer.

- TCP can best be modeled as a selective Repeat protocol.

- TCP does not support broadcasting but UDP supports broadcasting.

- ICMP is a local solution whereas TCP ack is a global solution which will nullify even if the ICMP is lost.

- A SYN segment cannot carry data, but it consumes one sequence number.

- A SYN+ACK segment cannot carry data, but it does consume one sequence number.

- An ACK segment, if carrying no data, consumes NO sequence number.

- The FIN segment consumes one sequence number if it doesn't carry any data.

- The FIN + ACK segment consumes only one sequence number if it does not carry data.

- ERTT = α*IRTT + (1- α)*NRTT

- Nagle’s solution for the silly window syndrome resolves the sender side problem and Clark's solution resolves the receiver side’s problem.

https://www.youtube.com/watch?v=SI6L8c8uPO4

- TCP Congestion Control https://www.geeksforgeeks.org/tcp-congestion-control/

- TCP Timers : https://www.geeksforgeeks.org/tcp-timers/

- Only those processes sending short messages, messages less than 65,507 bytes (65535 minus 8 bytes for the UDP header and minus 20 bytes for the IP header) can use UDP. The process can not expect from UDP to chop the data into different datagrams.

- In UDP if the sender decides not to include the checksum.

- Routing information protocol (RIP) uses UDP.

- The computational overhead in link state protocols is higher than in distance vector protocols. Bcz LSR is based upon global knowledge whereas DVR is based upon Local info .

- In case of slow start phase, cwnd size increases by 1 MSS on every successful acknowledgement and cwnd size approximately doubles on every RTT. GATE CSE 2018 | Question: 14 - GATE Overflow for GATE CSE

- Hamming code can correct single bit errors.

- Parity scheme can detect only odd no of errors, even no of errors cannot be detected if noise modifies.

- ARP request is broadcast and ARP reply is unicast.

- ARP request is never generated for a device of another network.

- CRC Properties : Computer Networks: GATE CSE 2009 | Question: 48 (gateoverflow.in)

- The TCP checksum includes the Pseudo IP header + TCP header + TCP body

- Parity check can detect odd no of errors but not even no of errors.

- Linear block codes is the group of codewords which the XOR of any two valid codewords is also a valid codeword.

- When the receiver receives a corrupted packet, there are two choices :

1) The receiver will silently discard the packet : In this case when the sender didn't get the ack for this packet until the timeout timer expires, then the sender will resend this packet.

2) The receiver will send negative ack to the sender about the corrupted packet. The advantage of negative ack is that the sender will retransmit immediately after getting negative ack without waiting for the timeout timer to expire. The negative ack is an optional functionality that can be added to improve the performance.

- ARP can only be used within a network (in the ethernet coverage).

We can also use ARP in two hosts which are in different subnets of the same network and connected through a router(proxy ARP) but we can not use ARP within two hosts that are on different networks.

- RARP : there is a dedicated server inside each subnet for assigning IP addresses to the host in that network using a mapping table.

- BOOTP: No need of having a dedicated server in each subnet. We use rely agents(routers having application layer functionalities) that forwards the request of a host to the BOOTP server. The BOOTP protocol provides static IP addresses (fixed).

- DHCP : Same as BOOTP but provides dynamic IP addresses using expiry timer. DHCP is backward compatible with BOOTP (dukan wahi hai bas maalik change ho gaya).

- IP protocol only supports unicasting. For multicasting it uses IGMP.

- https://gateoverflow.in/387815/go-classes-test-series-2023-mock-gate-test-1-question-36

GATE CSE 2020 | Question: 38 - GATE Overflow for GATE CSE

GATE IT 2008 | Question: 85 - GATE Overflow for GATE CSE

GATE CSE 2004 | Question: 57 - GATE Overflow for GATE CSE

GATE CSE 2012 | Question: 23 - GATE Overflow for GATE CSE

GATE CSE 2013 | Question: 37 - GATE Overflow for GATE CSE

GATE CSE 2017 Set 2 | Question: 20 - GATE Overflow for GATE CSE

GATE CSE 2014 Set 3 | Question: 27 - GATE Overflow for GATE CSE

Good question !

Question on MTU and offset :

GATE CSE 2018 | Question: 54 - GATE Overflow for GATE CSE

GATE CSE 2006 | Question: 46 - GATE Overflow for GATE CSE

GATE CSE 2007 | Question: 69 - GATE Overflow for GATE CSE

Good concept, Must read discussion :

GATE CSE 2009 | Question: 57, ISRO2016-75 - GATE Overflow for GATE CSE

GATE CSE 2009 | Question: 58 - GATE Overflow for GATE CSE

GATE CSE 2014 Set 1 | Question: 28 - GATE Overflow for GATE CSE

Here satellite communication is used but you don't need to consider 4xTp delay because there is a connection between a computer and a satellite and not between two ground stations that are connected via a satellite. Understand the difference !

GATE CSE 2016 Set 2 | Question: 55 - GATE Overflow for GATE CSE

This is the example for second case :

GATE IT 2008 | Question: 64 - GATE Overflow for GATE CSE

GATE IT 2006 | Question: 64 - GATE Overflow for GATE CSE

GATE CSE 2017 Set 1 | Question: 45 - GATE Overflow for GATE CSE

We need to consider two way  processing delays ,  one from sender side another from receiver side .

GATE IT 2006 | Question: 68 - GATE Overflow for GATE CSE

If the probability of  a frame reaching safely is Q, then mean no of transmissions = 1/Q.

GATE CSE 2011 | Question: 53 - GATE Overflow for GATE CSE

Based on distance vector routing count to infinity !

GATE CSE 2014 Set 3 | Question: 25 - GATE Overflow for GATE CSE

Good question on probability :

GATE CSE 2007 | Question: 65 - GATE Overflow for GATE CSE

GATE CSE 2019 | Question: 49 - GATE Overflow for GATE CSE

GATE IT 2004 | Question: 27 - GATE Overflow for GATE CSE

GATE IT 2005 | Question: 28 - GATE Overflow for GATE CSE

Very nice concept must read the second answer on GO.

GATE IT 2006 | Question: 66 - GATE Overflow for GATE CSE

Ambiguous question but must read the solution by Arjun Sir : GATE CSE 2009 | Question: 47 - GATE Overflow for GATE CSE

GATE CSE 2015 Set 1 | Question: 1‏9 - GATE Overflow for GATE CSE

Understand the 4th point !!!!!

(205) TCP Misc Part--1 - YouTube

Also study this : GATE CSE 2015 Set 2 | Question: 34 - GATE Overflow for GATE CSE

GATE CSE 2015 Set 3 | Question: 22 - GATE Overflow for GATE CSE

GATE CSE 2016 Set 2 | Question: 25 - GATE Overflow for GATE CSE

GATE CSE 2018 | Question: 25 - GATE Overflow for GATE CSE

Good questions :

GATE CSE 2021 Set 2 | Question: 7 - GATE Overflow for GATE CSE

GATE CSE 2021 Set 1 | Question: 45 - GATE Overflow for GATE CSE

Clear all doubts regarding Congestion window, receivers window and senders window size : https://gateoverflow.in/3836/gate-it-2005-question-73

https://gateoverflow.in/32145/find-the-size-of-senders-window-if

Go through with all the comments and answers …. Full clarity guaranteed

GATE IT 2008 | Question: 69 - GATE Overflow for GATE CSE

ACK form the Client is not Necessary for Connection Establishment, even the Data Packet Sent ( after the Lost ACK from Client) from the Client to Server can act as the ACK.

actually polling method is used to decide the station which is going to transmit next.....and

in this we use an algorithm to find a suitable station and then allocate Time equal to Tt+Tp ...

Once this time is over we ...again we use the same for other stations......

Hence, no. of channels doesn't matter for polling...

GATE IT 2007 | Question: 64 - GATE Overflow for GATE CSE

Concept of statistical TDM (backlogs) :

GATE IT 2007 | Question: 62 - GATE Overflow for GATE CSE

GATE CSE 2018 | Question: 14 - GATE Overflow for GATE CSE

Good question : GATE CSE 2018 | Question: 55 - GATE Overflow for GATE CSE

GATE IT 2005 | Question: 27 - GATE Overflow for GATE CSE

Computer Networks: GATE CSE 2009 | Question: 48 (gateoverflow.in)

Good analytical question :

Computer Networks: GATE IT 2008 | Question: 66 (gateoverflow.in)

Computer Networks: GATE CSE 2016 Set 1 | Question: 25 (gateoverflow.in)

Computer Networks: GATE IT 2005 | Question: 77 (gateoverflow.in)

Computer Networks: GATE IT 2006 | Question: 18 (gateoverflow.in)

Computer Networks: GATE CSE 2017 Set 2 | Question: 18 (gateoverflow.in)

Computer Networks: GATE IT 2006 | Question: 69 (gateoverflow.in)

Computer Networks: GATE CSE 2016 Set 2 | Question: 54 (gateoverflow.in)

Computer Networks: GATE CSE 2004 | Question: 54 (gateoverflow.in)

You got confused here for the value of k !!

The ‘k’ in exponential backoff algorithm is the kth time the collision happened for the same data.

Hell lot of confusion came while doing this !!!

Computer Networks: GATE CSE 1994 | Question: 9 (gateoverflow.in)

Good concept : Computer Networks: GATE IT 2005 | Question: 26 (gateoverflow.in)

Computer Networks: GATE CSE 2008 | Question: 59 (gateoverflow.in)

Beautiful question on token bucket :

Computer Networks: GATE CSE 2016 Set 1 | Question: 54 (gateoverflow.in)

The rate at which tokens are removed from the bucket is ALWAYS equal to the rate at which the packets are transmitted by the system.

The "Token Bucket" algorithm with a zero bucket capacity is nothing but the "Leaky Bucket" algorithm.

The host can send full throttle at the maximum transmission rate for a short while until it has drained the bucket.

When it (no. of tokens in the bucket) reaches zero, new packets can be sent only at the rate at which the buffer is filling; there can be no more bursts until the bucket has recovered.

Concept !! Receiver also decrements the TTL value.

Within a LAN, TTL is not decremented because inside LAN it never goes to the network layer, it is forwarded from the data link layer itself.

Once the receiver receives the packet it decrements the value of TTL and checks if >=0 then it accepts in case of router decrements and checks >0 it forwards it, else it discards it and sends an ICMP to the sender.

Computer Networks: GATE CSE 2014 Set 2 | Question: 25 (gateoverflow.in)

Computer Networks: GATE CSE 2005 | Question: 74 (gateoverflow.in)

Very good question :

Computer Networks: GATE IT 2004 | Question: 85 (gateoverflow.in)

Computer Networks: GATE IT 2005 | Question: 75 (gateoverflow.in)

Good question :

Computer Networks: GATE IT 2006 | Question: 67 (gateoverflow.in)

Computer Networks: GATE IT 2007 | Question: 69 (gateoverflow.in)

Computer Networks: GATE IT 2008 | Question: 68 (gateoverflow.in)

Fast retransmission is a congestion control algorithm that makes it possible for sender to quickly retransmit lost packets. Without fast retransmission, sender can retransmit only after timeout timer has expired. With fast transmission, reciever can send negative acknowledgements for the packets that it does not receive properly. If sender receives three negative acknowledgements for a packet, it will retransmit it without waiting for timeout timer.

Very good question :

Computer Networks: GATE CSE 2005 | Question: 73 (gateoverflow.in)

Dividing messages into packets decreases the transmission time due to pipelined transmission.

Computer Networks: GATE CSE 2014 Set 2 | Question: 26 (gateoverflow.in)

Computer Networks: GATE CSE 2015 Set 3 | Question: 36 (gateoverflow.in)

Computer Networks: GATE CSE 2021 Set 2 | Question: 54 (gateoverflow.in)

Here the throughput comes out to be 0.135 that means out of 1000 frames transmitted only 135 reached successfully.

Computer Networks: GATE CSE 2017 Set 2 | Question: 09 (gateoverflow.in)

RIP uses the UDP as its transport protocol.

OSPF needs to perform reliable multicasting because it needs to talk to multiple possible neighbours on the same network segment. Now, TCP does not support multicast and UDP is not reliable Therefore, OSPF implements its own transport mechanism that allows both for reliability (acknowledgements and retransmissions of lost segments) and multicasting, bypassing both TCP and UDP.

Good question on count to infinity problem :

Computer Networks: GATE IT 2005 | Question: 85b (gateoverflow.in)

Computer Networks: GATE IT 2007 | Question: 63 (gateoverflow.in)

Link state routing ka made easy k notes me kuch aur bataya hai, baki jagah kuch aur hi hai

Computer Networks: GATE IT 2008 | Question: 67 (gateoverflow.in)

Timeout = 2xRTT